SDK-2/3 Quartz timer switch

I. Feature:

The product imports advanced technology abroad, made from of imported quartz syntony machine and CMOS elements, time accurately and work stably, Which has been the latest items of auto time control instruments in the past few years and has received great popularity from domestic users, and well sold throughout the country.

II. Function:

The product can use directly or match with AC contactor, automatically switch single-phase or three-phase circuit at regular time for users to control all kinds of electrical apparatus, which is applicable for road lamp, neon lamp, broadbill, electric bell, oven, electric cooker bellows, three-peck power of electric supply dept. and automatic equipments, etc. The users can use it for automatically brake-jump and open or close power.

III. Main technical index:

1.       Work precision: ≤1s/day;

2.       Timer precision: ≤5 Min;

3.       Switch period of time: Min:1.5Hrs, Max: 24Hrs;

4.       Environmental temperature: -20℃～+50℃;

5.       Rated voltage AC contact: 100V、220V、380V;

6.       Rated current AC contact (resistant): 100V 7.5A、220V 5A、380V 2A;

7.       Power supply DC of quartz clock: 1.5V;

8.       Outline size: 190*143*75mm

IV. Instructions

1. Loosen the adjuster screws, fix the adjuster to the clock panel according to requirement, a pair of adjusters shall make up of a control time period for open and close. If you need one to two control periods every day, then you shall fix one to two pairs of adjusters. After fixing, unload the unwanted adjusters, and begin to adjust the present time to open. Take controlling the road lamp for example, it is planed to open the lamps at 19:00 P.M. and close at 5:00 A.M., you need to put the black adjusters to 19:00 and red adjusters to 5:00. The product shall supply power according to the setting time to open and close automatically.

2. Turn the clock panel clockwise (anti-clockwise strictly prohibited). First make the adjuster indicating the time to start to locate at the top right corner of black handle, and dial the hexangular cam anti-clockwise (anti-dial strictly prohibited), making the clock switch to be open or close according to requirements, and make one tooth of the cam point to the black handle (Meanwhile the adjuster pole has just dialed the cam to make the clock ON/OFF). When it reaches the first control time, install in with 1.5V batteries to the battery box, then the clock begin working.

V. Using notices

1. The poles of the 1.5V battery for quartz clock shall be not met in reverse, and you shall change at scheduled time, commonly one No.1 battery of high performance can use for 1 year.

2. Strictly prohibit anti-dialing panel and the hexangular cam, otherwise it will damage the machine and part.

3. After using for a long time, you shall readjust it, in order to clear up the errors.

4. The place for install the clock shall avoid insolation, strong cool, strong magnetic field, strong caustici gas or relative humidity not more than 85%.

5. If the clock is used for sensible load for bigger power motors, the rated current value or resistant load shall match with AC contactor.